# GeoTIFF Format

# for IRS

# Digital Data Products

July 2002

Data Products Software Division

Signal and Image Processing Group

SAC/ ISRO

### AHMEDABAD – 380 015

Government of India

Space Applications Centre (ISRO)

Ahmedabad-380 015

Document Control and Data Sheet

1.	Document No & Date	:	IRS/DP/SAC/SIIPA/SIPG/TN-07 /July 2002

2.	Title		:	GeoTIFF format for IRS Digital 		            Data Products

3.	Type of Document		:	Technical

4.	No. of pages			:	25

5.	Authors			:	IRS  DP Team					Compiled by 			:	Sk.Sazid Mahammad

6.	Originating Unit	:	Signal & Image Processing Group, Signal, Image & Information Processing Area

7.	Security Classification	:	Unrestricted

8.	Distribution	:	To identified individuals within ISRO/DOS and IRS Data Users

5NTENTS

Page No

1.0  Introduction 								4

2.0  TIFF 									5

2.1  Overview of TIFF File Structure 				5
            2.2  Baseline TIFF 							10
            2.3  Extended TIFF 							10

3.0  Brief description GeoTIFF

3.1	Basic Features						15

3.2	GeoTIFF System/Software requirements			16

3.3	GeoTIFF File and “Key” structure Hierarchy		16

3.4	Where to get GeoTIFF specification 				20

4.0 IRS - DPS Usage 								

             4.1  File naming conventions. 					21
            4.2  Ellipsoids and Map projections supported. 			22
            4.3  Contents of a typical IRS GeoTIFF product. 			24

# 1.0 Introduction

The spaceborn remote sensing images in digital form has gained wide spread popularity, over the last decade with the advances in the field of Digital Image Processing and Geographical Information System (GIS) and evolution of computer hardware and software.  Although currently various data formats  (e.g. PGM, GIF, BMP, TIFF) are in use for storage of raster image data, they have a common limitation in cartographic applications. The main problem is that, it is almost impossible to store any geographic information together with image data in a unified and well-defined way in the above mentioned formats.  There was a requirement to develop a new standard or the extension of an existing one, by adding a formalism to store information about geo-location of the image, the underlying cartographic model, a set of tie points and other relevant data.  This new format was required to have platform independency, flexibility and extensibility. The  Aldus-Adobe's public domain Tagged Image File Format (TIFF) is one of the widely used raster file formats, which is platform independent and has provision for extension.  In 1990 one positive initiative was started by the Intergraph Corp. and the JPL Cartographic Working Group, based on the TIFF standard. The basic idea was to exploit the extensibility feature of TIFF which allows to officially register new TIFF Tags in order to create a well established structured format/space for a variety of geographic information.

Though it is very difficult to standardize remote sensing satellite digital image data products' formats and its contents, GeoTIFF format is an effort for this considering widespread application of GIS packages worldwide. The basic idea behind this file format is to supplement the bare image data with all information necessary for the transformation from image space into a geographic or cartographic coordinate system.

This document has been written keeping IRS series Data Products contents in GeoTIFF format. The aim of this document is not to give  complete TIFF or GeoTIFF specification, but only the fields used to give details of a given IRS data product. For a full understanding of GeoTIFF, this document has to be read along with the original GeoTIFF (Revision 1.0) specification.  The GeoTIFF specification can be down loaded from the site given in chapter 3.

# 2.0 TIFF (Tagged Image File Format)

TIFF  (Tagged Image File Format), is a tag-based file format for storing and interchanging raster images. The first version of TIFF specification was published by Aldus Corporation in 1986, after a series of meetings with various scanner manufactures and software developers. TIFF is to describe and store raster image data. The main advantages of TIFF is its suitability for a wide range of applications and its independence of computer's architecture, operating system, and graphics hardware. It is reasonably compact and handles black-and-white, grayscale, and color images, allowing a user to adjust for the unique characteristics of a scanner, monitor, or printer. TIFF allows color resolution up to 48 bits (a 16-bit field each for R, G, and B), either as full RGB color or in a 64k-color palette .The TIFF 6.0 specification, released in June 1992 is taken as reference by GeoTIFF.

There are two variants in revision 6.0 of TIFF: baseline and extended TIFF. Within both these variants, there are many optional capabilities. Furthermore, TIFF data vary according to photometric type and compression method. All TIFF 6.0 readers and writers must handle the baseline set of data fields. The way in which these data fields are handled, depends on  whether they are mandatory or optional.

## 2.1 Overview of TIFF File Structure

The TIFF format has a three-level hierarchy. From highest to lowest, the levels are:

1) A file Header.

2) One or more directories called IFDs, containing codes and their data, or pointer to the data.

3) Data.

The graphical view of TIFF file structure is given in Fig.1.

The File Header

The TIFF file begins with an 8-byte header which gives basic information about the file such as byte order (Little Endian or Big Endian), TIFF file ID or Version Number (which is always 42) and a pointer to first Image File Directory (IFD). The structure of TIFF file header is as follows:

TABLE  : 1	TIFF  file Header

| Offset | Length | Description |
| --- | --- | --- |
| 0 | 2 | Byte Order: MM or II |
| 2 | 2 | "Version number ": which is always 42 |
| 4 | 4 | Pointer to first IFD |

*  Byte Order Field :  The first header entry is in byte counts 0 and 1. It is a two-letter ASCII record containing the codes for the letters MM or II (4D4D hex or 4949 hex, respectively). MM stands for Motorola architecture, signifying that bytes comprising 16, 32 0r 64 bit numbers are stored in the order of most-to-least significant (big endian). II denotes file created in the Intel Architecture, where bytes are stored in the least-to-most significant order (little endian). By reading this record , a Mac or UNIX workstation-based file reader can detect a file created on a PC or vice versa, and interpret it properly.

* Version field  :  The second header entry, a two byte number termed Version in the specification, actually has nothing to do with any version number or with the revision number of the specification. It is always the decimal number 42 (2A hex). For practical purposes, this code identifies the file as a TIFF file.

* First IFD Offset pointer : The final header entry is a 4-byte pointer to the location of the first Image File Directory (IFD). The order of bytes here, as elsewhere, depends on the MM or II designation in the first entry. This pointer begins at an offset of 4, the fifth byte in the file.

Image File Directories (IFDs)

Most Likely, the next structure in the file after the header will be the first (or only) IFD, but not necessarily. From here on, everything is found by following pointers. So, to locate the first IFDuse the header's pointer.

An IFD consists of 12-byte entries, typically tagged pointers. The structure of an IFD and its entries are as follows:

TABLE : 2	TIFF IFD Structure

| Offset | Length | Description |
| --- | --- | --- |
| 0 | 2 | Entry Count |
| 2 | 12 | Entry 0 |
| 14 | 12 | Entry 1 |
| . | . | . |
| . | . | . |
| n*12 +2 | 12 | Entry n |
| n*12 + 2 + 12 | 4 | Pointer to subsequent IFD, if any, or 0000 |

* Entry Count field : Since there may be any number of  IFD entries, the first field in an IFD is a 2-byte count of number of entries.

* IFD entries : IFD entries are 12-byte fields, beginning with the tag code that specifies what type of data are present. The entries are ordered numerically within the IFD, according to the tag numbers, a feature that helps TIFF readers quickly determine which fields are present and which are not.

* Terminating field : The last entry in an IFD will be four bytes of zero, unless there is more than one IFD. If there is more than one IFD, the last entry of the preceding IFD contains a 4-byte pointer to the next one.

TABLE : 3	TIFF IFD Entry Structure

| Offset | Length | Description |
| --- | --- | --- |
| 0 | 2 | Tag |
| 2 | 2 | Type of Data |
| 4 | 4 | Count field |
| 8 | 4 | Data pointer or data field |

* Tag : The first two bytes are tag, which , if public , may be looked up in the specification. These codes are assigned by the TIFF administrator ( Aldus Developer's Desk), in blocks of five.

* Type code : The next two bytes comprise a code indicating the type of data in the pointed field. Note that some tags allow a variety types, so TIFF readers should always check this code. TIFF 6.0 supports a veriety of data types ; the codes, the data types, and their names are as follows :

TABLE : 4	TIFF 6.0 DATA TYPES

| Data Type Code | No of Bytes | Standard Data Type |
| --- | --- | --- |
| 1 | 1-byte integer | type BYTE |
| 2 | 1-byte integer, ASCII | type ASCII |
| 3 | 2-byte integer | type SHORT |
| 4 | 4-byte integer | type LONG |
| 5 | 8-byte fraction | type RATIONAL (4-byte numerator followed by 4-byte denominator) |
| 6 | 1-byte signed integer | type SBYTE |
| 7 | 1-byte of anything | type UNDEFINED, e.g. a pointer to a complex data structure. |
| 8 | 2-byte signed integer | type SSHORT |
| 9 | 4-byte signed long | type SLONG |
| 10 | 8-byte signed fraction | type SRATIONAL, SLONG numerator, SLONG denominator. |
| 11 | 4-byte floating point | type FLOAT, IEEE format |
| 12 | 8-byte floating point | type DOUBLE, double-precision IEEE |

* Count field :  The 4-byte field specifies the number of values in the data field, not the number of bytes. The number of bytes  can be computed  by multiplying the count by the number of bytes in the data type. For example, a count of 64 with a data type of LONG means there will be 256 bytes. ( The count was referred to as the length in earlier revisions of TIFF.)

* Data pointer or  data field : The final four bytes are usually a pointer to the start of a data field. Sometimes, however, this field contains not a pointer, but the actual data. If there are four bytes of data or less, as computed by the product of count and the byte count indicated by type, then the field contains data. Otherwise, the field is a pointer ( the offset in bytes from the start of the file to the start of the data field).

2.2  Baseline TIFF

Baseline  TIFF  specification mostly affects TIFF readers. TIFF readers must be able to process all the fields given in baseline table or must assume the specified default, if the field is not present, regardless of the image-type expected. TIFF writers must include all fields required for the type of image data written, except where the default value is desired.  Refer Table 5 and Table 6 for details of TIFF 6.0 Tags.

2.3  Extended TIFF

TIFF 6.0 defines numerous extensions. These allow TIFF to offer the following general capabilities.

*	Alternative compression schemes : CCITT, LZW(with and without   differencing), and JPEG.

*	Alternative color-representation schemes: CMYK, YCbCr, and CIE L*a*b*.

*	Image-quality enhancements : Halftone "hinting," RGB colorimetry.

*	Special image effects : Alpha data ( mating, masks and overlays), tiled  images.

*	Document storage and retrieval aids: document and page names, page numbers, and position on the page.

These extensions take the form of additional tags and/or special codes for baseline tags.

# Baseline TIFF Tags its usage in IRS DPS

IRS-DPS uses Grayscale model to store image data. For  each band of data a separate file with band name is given. In the following table the relevant/used fields of TIFF in context of IRS has been indicated.

Following fields may change or contain no information as per changes in DPS s/w and team members.

Extra Fields.

Software(305), DateTime(306), Artist(315), HostComputer(316).

Mandetory/required fields and their defaults. X indicates field must be written  in TIFF file for given image type(Bi-level,Grayscale,Palette,RGB).

TABLE : 5

| Field /Tag Name | TagValue | Default | Bi-level | Grayscale | Palette | RGB | IRS-1C/1D/P6( DPSUSE) Grayscale/RGB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ImageWidth | 256 | none | X | X | X | X | Yes |
| ImageLength | 257 | none | X | X | X | X | Yes |
| BitsPerSample | 258 | 1 |  | X | X | X | Yes |
| ColorMap | 320 | none |  |  | X |  | NotApplicable |
| Compression | 259 | 1 | X | X | X | X | Yes |
| PhotometricInterpretation | 262 | none | X | X | X | X | Yes |
| StripOffsets | 273 | none | X | X | X | X | Yes |
| RowsPerStrip | 278 | (2**32)-1 | X | X | X | X | Yes |
| StripByteCounts | 279 | none | X | X | X | X | Yes |
| Xresolution | 282 | none | X | X | X | X | Yes |
| Yresolution | 283 | none | X | X | X | X | Yes |
| ResolutionUnit | 296 | 2 | X | X | X | X | Yes |

All Baseline field s/tags of  TIFF 6.0 and IRS DPS Usage.

TABLE : 6

| Tag Name | TagValue | IRS-1C/1D (DPSUse ) | Remarks in IRS DPS Context |
| --- | --- | --- | --- |
| NewSubFileType | 254 | No |  |
| SubFileType | 255 | No |  |
| ImageWidth | 256 | Yes | Gives PixelsPerScanline or RecordLength. |
| ImageLength | 257 | Yes | Gives No of ScanLines in the given image. |
| BitsPerSample | 258 | Yes | For IRS DPS this is 8 since image data is 8bit. |
| Compression | 259 | Yes | Code 1 is used in DPS showing no compression |
| PhotometricInterpretation | 262 | Yes | 1: in GrayScale 2: in RGB |
| Thresholding | 263 | No |  |
| CellWidth | 264 | No |  |
| CellLength | 265 | No |  |
| FillOrder | 266 | No |  |
| DocumentName | 269 | Yes |  |
| ImageDescription | 270 | Yes | Gives EOSAT/ISRO Fast Format Header |
| Make | 271 | No |  |
| Model | 272 | No |  |
| StripOffsets | 273 | Yes |  |
| Orientation | 274 | Yes | Code 1 is used : FisrtRowOnTop &FirstColumnOnLeft |
| SamplesPerPixel | 277 | No: GrayScale Yes : in RGB | GrayScale : NA RGB : 3 |
| RowsPerStrip | 278 | Yes |  |
| StripByteCounts | 279 | Yes |  |
| MinSampleValue | 280 | Yes | Minimum possible gray value ( "0" for IRS) |
| MaxSampleValue | 281 | Yes | Maximum possible Gray Value ("255" for IRS) |
| Xresolution | 282 | Yes | Number of pixels in one ResolutionUnit |
| Yresolution | 283 | Yes | No Of ScanLines in one ResolutionUnit |
| PlanarConfiguration | 284 | No | data i/o |
| PageName | 285 | No |  |
| Xposition | 286 | No |  |
| Yposition | 287 | No |  |
| FreeOffsets | 288 | No |  |
| FreeByteCounts | 289 | No |  |
| GrayResponseUnit | 290 | No |  |
| GrayResponseCurve | 291 | No |  |
| Group3Options | 292 | No |  |
| Group4Options | 293 | No |  |
| ResolutionUnit | 296 | Yes | Code 3 is used to indicate unit as Centimeters. |
| PageNumber | 297 | No |  |
| ColorResponseUnit | 300 | No |  |
| TransferFunction | 301 | No |  |
| Software | 305 | Yes | Software Release Number/Description. |
| DateTime | 306 | Yes | Date and Time of Product generation. |
| Artist | 315 | Yes | Authors name and organization name is give. |
| HostComputer | 316 | Yes | Processing System/Center Name is given. |
| Predictor | 317 | No |  |
| WhitePoint | 318 | No |  |
| PrimaryChromacities | 319 | No |  |
| ColorMap | 320 | No |  |
| TileWidth | 322 | No |  |
| TileLength | 323 | No |  |
| TileOffsets | 324 | No |  |
| TileByteCounts | 325 | No |  |
| BadFaxLines | 326 | No |  |
| CleanFaxData | 327 | No |  |
| ConsecutiveBadFaxLines | 328 | No |  |
| SubIFD | 330 | No |  |
| InkSet | 332 | No |  |
| InkNames | 333 | No |  |
| DotRange | 336 | No |  |
| TargetPrinter | 337 | No |  |
| ExtraSamples | 338 | No |  |
| SampleFormat | 339 | No |  |
| SminSampleValue | 340 | No |  |
| SmaxSampleValue | 341 | No |  |
| JPEGTables | 347 | No | used by JPEG codec |
| YcbCrCoefficients | 529 | No | used by TIFFReadRGBAImage support |
| YcbCrSubsampling | 530 | No | tile/strip size calculations |
| YcbCrPositioning | 531 | No |  |
| ReferenceBlackWhite | 532 | No |  |
| Matteing | 32995 | No | none (obsoleted by ExtraSamples tag) |
| DataType | 32996 | No | none (obsoleted by SampleFormat tag) |
| ImageDepth | 32997 | No | tile/strip calculations |
| TileDepth | 32998 | No | tile/strip calculations |

# 3.0 Brief Description of GeoTIFF

The GeoTIFF specification defines a set of TIFF tags provided to describe all "Cartographic" information associated with TIFF imagery that originates from satellite imaging systems, scanned aerial photography, scanned maps, digital elevation models, or as a result of geographic analyses. Its aim is to allow means for tying a raster image to a known model space or map projection.

GeoTIFF does not intend to become a replacement for existing geographic data interchange standards, such as the USGS SDTS standard or the FGDC metadata standard. Rather, it aims to augment an existing popular 
raster-data format to support georeferencing and geocoding information.

3.1 Basic Features

GeoTIFF format  fully complies with the TIFF 6.0 specifications, and its extensions do not in any way go against the TIFF recommendations, nor do they limit the scope of raster data supported by TIFF.

It uses a small set of reserved TIFF tags to store a broad range of georeferencing information, catering to geographic as well as projected coordinate systems needs. Projections include UTM, US State Plane and 
National Grids, as well as the underlying projection types such as Transverse Mercator, Lambert Conformal Conic, etc.

It uses a "MetaTag" (GeoKey) approach to encode dozens of information elements into just 6 tags, taking advantage of TIFF platform-independent data format representation to avoid cross-platform interchange difficulties. These keys are designed in a manner parallel to standard TIFF tags, and closely follow the TIFF discipline in their structure and layout. New keys may be defined as needs arise, within the current framework, and without requiring the allocation of new tags from Aldus/Adobe.

GeoTIFF format uses numerical codes to describe projection types, coordinate systems, datums, ellipsoids, etc. The projection, datums and ellipsoid codes are derived from the EPSG list compiled by the Petrotechnical Open Software Corporation (POSC), and mechanisms for adding further international projections, datums and ellipsoids has been established. The GeoTIFF information content is designed to be compatible with the data decomposition approach used by the National Spatial Data Infrastructure (NSDI) of the U.S. Federal Geographic Data Committee (FGDC).

3.2 GeoTIFF System/Software Requirements

GeoTIFF requires support for all documented TIFF 6.0 tag data-types, and in particular requires the IEEE double-precision floating point "DOUBLE" type tag. Most of the parameters for georeferencing will not have 
sufficient accuracy with single-precision IEEE, nor with RATIONAL format storage. The only other alternative for storing high-precision values would be to encode as ASCII, but this does not conform to TIFF recommendations for data encoding.

It is worth emphasizing here that the TIFF spec indicates that TIFF-compliant readers shall honor the 'byte-order' indicator, meaning that 4-byte integers from files created on opposite order machines will be swapped in software, and that 8-byte DOUBLE's will be 8-byte swapped.

A GeoTIFF reader/writer, in addition to supporting the standard TIFF tag types, must also have an additional module which can parse the "Geokey" MetaTag information

3.3 GeoTIFF File and "Key" Structure Hierarchy

This gives the abstract file-format and "GeoKey" data storage mechanism used in GeoTIFF. To see graphical view of the GeoTIFF file structure please refer Fig. 2.

A GeoTIFF file is a TIFF 6.0 file, and inherits the file structure as described in the corresponding portion of the TIFF spec. All GeoTIFF specific information is encoded in several additional reserved TIFF tags, and contains no private Image File Directories (IFD's), binary structures or other private information invisible to standard TIFF readers.

The number and type of parameters that would be required to describe most popular projection types would, if implemented as separate TIFF tags, likely require dozens or even hundred of tags, exhausting the limited resources of the TIFF tag-space. On the other hand, a private IFD, while providing thousands of free tags, is limited in that its tag- values are invisible to non-savvy TIFF readers (which don't know that the IFD_OFFSET tag value points to a private IFD).

To avoid these problems, a GeoTIFF file stores projection parameters in a set of "Keys" which are virtually identical in function to a "Tag", but has one more level of abstraction above TIFF. Effectively, it is a sort of "Meta-Tag". A Key works with formatted tag-values of a TIFF file the way that a TIFF file deals with the raw bytes of a data file. Like a tag, a Key has an ID number ranging from 0 to 65535, but unlike TIFF tags, all key ID's are available for use in GeoTIFF parameter definitions.

The Keys in GeoTIFF (also call "GeoKeys") are all referenced from the GeoKeyDirectoryTag, which defined as follows:

GeoKeyDirectoryTag: 
      	Tag 	= 34735 (87AF.H) 
      	Type 	= SHORT (2-byte unsigned short) 
      	N 	= variable, >= 4 
      	Alias	: ProjectionInfoTag, CoordSystemInfoTag 
      	Owner: SPOT Image, Inc.

This tag may be used to store the GeoKey DirecGeoKey directory header information. The header values consist of the following information, in order:

Header={KeyDirectoryVersion, KeyRevision, MinorRevision, NumberOfKeys}

where

"KeyDirectoryVersion" indicates the current version of Key  implementation, and will only change if this Tag's Key  structure is changed. (Similar to the TIFFVersion (42)).  The current DirectoryVersion number is 1. This value will most likely never change, and may be used to ensure that this is a valid Key-implementation.

"KeyRevision" indicates what revision of Key-Sets are used.

"MinorRevision" indicates what set of Key-codes are used. The complete revision number is denoted <KeyRevision>.<MinorRevision>

"NumberOfKeys" indicates how many Keys are defined by the rest  of this Tag.

This header is immediately followed by a collection of <NumberOfKeys>  KeyEntry sets, each of which is also 4-SHORTS long. Each KeyEntry is modeled on the "TIFFEntry" format of the TIFF directory header, and is 
of the form:

KeyEntry = { KeyID, TIFFTagLocation, Count, Value_Offset }

where

"KeyID" gives the key-ID value of the Key (identical in function  to TIFF tag ID, but completely independent of TIFF tag-space),

"TIFFTagLocation" indicates which TIFF tag contains the value(s)  of the Key: if TIFFTagLocation is 0, then the value is SHORT,   and is contained in the "Value_Offset" entry. Otherwise, the type  (format) of the value is implied by the TIFF-Type of the tag   containing the value.

"Count" indicates the number of values in this key.

"Value_Offset" Value_Offset indicates the index- offset *into* the TagArray indicated by TIFFTagLocation, if 
      it is nonzero. If TIFFTagLocation=0, then Value_Offset  contains the actual (SHORT) value of the Key, and 
      Count=1 is implied. Note that the offset is not a byte-offset,   but rather an index based on the natural data type of the 
      specified tag array.

Following the KeyEntry definitions, the KeyDirectory tag may also  contain additional values. For example, if a Key requires multiple SHORT  values, they shall be placed at the end of this tag, and the KeyEntry will set TIFFTagLocation=GeoKeyDirectoryTag, with the Value_Offset  pointing to the location of the value(s).

All key-values which are not of type SHORT are to be stored in one of  the following two tags, based on their format:tory, which defines and references the "GeoKeys", as described below.

The tag is an array of unsigned SHORT values, which are primarily  grouped into blocks of 4. The first 4 values are special, and contain

GeoDoubleParamsTag:

Tag 	= 34736 (87BO.H) 
      	Type 	= DOUBLE (IEEE Double precision) 
      	N 	= variable 
      	Owner: SPOT Image, Inc.

This tag is used to store all of the DOUBLE valued GeoKeys, referenced  by the GeoKeyDirectoryTag. The meaning of any value of this double array  is determined from the GeoKeyDirectoryTag reference pointing to it.  FLOAT values should first be converted to DOUBLE and stored here.

GeoAsciiParamsTag:

Tag 	= 34737 (87B1.H) 
      	Type 	= ASCII 
      	Owner: SPOT Image, Inc. 
      	N 	= variable

This tag is used to store all of the ASCII valued GeoKeys, referenced by  the GeoKeyDirectoryTag. Since keys use offsets into tags, any special  comments may be placed at the beginning of this tag. For the most part,  the only keys that are ASCII valued are "Citation" keys, giving  documentation and references for obscure projections, datums, etc.

Note on ASCII Keys:

Special handling is required for ASCII-valued keys. While it is true  that TIFF 6.0 permits multiple NULL-delimited strings within a single  ASCII tag, the secondary strings might not appear in the output of naive "tiffdump" programs. For this reason, the null delimiter of each ASCII  Key value shall be converted to a "|" (pipe) character before being  installed back into the ASCII holding tag, so that a dump of the tag  will look like this.

AsciiTag="first_value|second_value|etc...last_value|"

A baseline GeoTIFF-reader must check for and convert the final "|" pipe  character of a key back into a NULL before returning it to the client  software.

3.4 Where to get GeoTIFF Specification

Following are the sites where details of GeoTIFF spec. is available.

EPSG/POSC tables, and source code is available via anonymous FTP at:

ftp://mtritter.jpl.nasa.gov/pub/tiff/geotiff/ 
and is mirrored at the USGS:

ftp://ftpmcmc.cr.usgs.gov/release/geotiff/jpl_mirror/

There are several subdirectories called spec/ tables/ and code/.

The USGS also has an archive of prototype GeoTIFF images at:

ftp://ftpmcmc.cr.usgs.gov/release/geotiff/images/

Information and a hypertext version of the GeoTIFF spec is available via WWW at the following site:

http://www-mipl.jpl.nasa.gov/cartlab/geotiff/geotiff.html

A mailing-list is currently active to discuss the on-going development of this standard. To subscribe to this list, send e-mail to:

GeoTIFF-request@tazboy.jpl.nasa.gov

with no subject and the body of the message reading:

subscribe geotiff  your-name-here

To post inquiries directly to the list, send email to:

geotiff@tazboy.jpl.nasa.gov

# 4.0. IRS DPS  Usage : Both TIFF and GeoTIFF Conventions

Following file naming conventions are used in Data Products of IRS – 1C/1D

4.1 File Naming Convention

IRS -1C/1D has three sensors namely LISS-3, PAN and WiFS.

IRS – P6 has also three sensors namely LISS-3, LISS-4, AWiFS.

Liss-3, LISS-4,WiFS , AWiFS

BANDn.tif where     n =2,3,4,5 for LISS-3 , AWiFS,

= 2,3,4 for LISS-4.

= 3,4 for WiFS.

For example a 4 band LISS-3/AWiFS product in Gray scale will contain following four files:

BAND2.tif 
    BAND3.tif 
    BAND4.tif 
    BAND5.tif

In case of RGB model only three bands 2, 3, and 4 can be choosen. So a single tiff file will be generated by the name BAND_RGB.tif. In RGB model image data is stored in BIP(Band Interleaved by Pixel) format.

A typical WiFS product will contain following file.

BAND3.tif 
    BAND4.tif

PAN 
    BAND.tif .

4.2 Ellipsoid and Map Projection supported

IRS-DPS uses Grayscale model to store image data. For  each band of data a separate file with band name is given.

IRS Data Product supports various Ellipsoids and map projections.

Map Projections

Following map projections are supported in IRS-DPS.

Projection Name                                        	Mnemonic

Universal Transverse Mercator                  	UTM 
             State Plane Coordinate System                  	SPCS 
             Albers Conical Equal Area                           	ACEA 
             Lambett's Conformal Conic                        	LCC 
             Mercator                                                          MER 
             Polar Stereographic                                       	PS 
             Polyconic                                                        PC 
             Equidistant Conic (Type A & B)                  	EC 
             Transverse Mercator (Gauss-Krueger)     	TM 
              Stereographic                                                 SG 
             Lamberts Azimuthal Equal Area                 	LAEA 
             Azimuthal Equidistant                                  	AE 
             Gnomonic                                                       GNO 
             Orthographic                                                   OG 
             General Vertical Near-Side Perspective  	GVNP 
             Sinusoidal                                                       SIN 
             Equirectangular (Plate Carree)                      	ER 
             Miller Cylindrical                                           MC 
             Van Der Grintern I                                         VDG 
             Oblique Mercator (Type A & B)                    OM 
             Space Oblique Mercator                                 SOM

Earth Ellipsoids

This appendix contains the earth ellipsoids used in products. 
      This list of ellipsoids  shows the name and the identifier used in Record 
        3.  Field 3 of the header file.

Ellipsoid Name  Semi-Major Axis  Semi-Minor Axis  Mnemonic 
                         	(meters)                  (meters)

Clarke 1866           6378206.400000      6356583.800000  CLARKE_1866 
Clarke 1880           6378249.145000      6356514.869550   CLARKE_1880 
International1967   63778157.50000      6356772.200000    INTERNATL_1967 
International          6378388.000000      6356911.946130    INTERNATL_1909

WGS 66                6378145.000000     6356759.769356     WGS_66 
WGS 72                6378135.000000     6356750.519915     WGS_72

WGS 84                6378137.000000     6356752.314000     WGS_84
GRS 1980             6378137.000000     6356752.314140     GRS_80 
Airy                     6377563.396000     6356256.910000     AIRY 
Modified Airy       6377340.189000     6356034.448000     MODIFIED_AIRY 
Everest                 6377276.345200     6356075.413300     EVEREST 
Modified Everest   6377304.063000     6356103.039000     MODIFIED_EVEREST 
Mercury 1960       6378166.000000     6356784.283666     MERCURY_1960 
Modified Mercury 6378150.000000     6356768.337303     MOD_MERC_1968 
Bessel                   6377397.155000    6356078.962840     BESSEL 
Walbeck               6376896.000000     6355834.846700     WALBECK 
Southeast Asia      6378155.000000     6356773.320500     SOUTHEAST_ASIA 
Australian Natl.     6378160.000000     6356774.719000     AUSTRALIAN_NATL 
Krassovsky           6378245.000000     6356863.018800    KRASSOVSKY 
Hough                  6378270.000000     6356794.343479    HOUGH 
6370997 Sphere    6370997.000000     6370997.000000     6370997_M_SPHERE

4.3  Contents of a typical  IRS GeoTIFF Product

The content of a  Geocoded product in GeoTIFF format and various fields are shown below.

The various fields of TIFF used by IRS Data Products are as follows:

The various fields of GeoTIFF used by IRS Data Products are as follows: